Orf 467 

Transportation Systems Analysis

Fall 2007


Class #9
Part 2:  Planning and Analysis Tools of Transportation Demand and Investment

Modal Split: MM 5.4.3 pp282-3, MM 5.5 pp 290-303
· Direct generation usage models
· These models attempt to forecast mode usage as a function of land use and socio-economic characteristics of the trip production area/zone.  They focus on auto ownership (or lack thereof ) and relegate relatively little mode share to transit.  These are generally derived from regression analyses that ‘correlates” know auto ownership rates and mass transit patronage rates to the land use and socio economic characteristics of a zone.  They suggest that a change in mode share is dominated by these variables rather than the relative quality of the competing modes.  In the typical urban/suburban area, mass transit has been such a poor competitor that unless there is a radical change in the quality of mass transit, these relationships have been good at forecasting changes in mode share as the land use and socio economic conditions of an area changes.  I find these models intellectually unsatisfying because they don’t reflect changes in service in the modes themselves.

· Trip – Interchange node usage models.  These models focus on the attributes of each mode in the service of each trip.  They basically model the decision process that a traveler makes in choosing to go by car, bus train, walk etc.   Each mode is characterized in terms of its Utility and a choice process is modeled.  That process  could be a “winner takes all” or some softening of that choice that suggests that the “winner” only gets a higher share but that the losers also “win” some.
Logit Model:  Given a set of alternatives, Pit , the Probability of an individual t choosing alternative i from a set of alternatives Pjt is:

Pit = P(Uit(Ujt)  for every j in Cj


Where   Ujt =Vjt + εjt  (The Utility, from the perspective of the decider, Ujt , is made up of the Utility, from the perspective of the modeler, Vjt , plus some error term εjt)
If we assume (largely for the sake of analytical convenience) that the error term is “independently and identically distributed with a Gumbel Type I distributions” whose cumulative distribution function is given by:  F(w) = e-e-w

Then it can be shown that 
Pit = eVit /  (j=1,n eVjt
Where  

 

Pit  =  probability of choosing mode i by individual t
n    =  number of modes being considered 

Vi   =  is the modeled Utility function of mode i for individual t . The simplest form of which is a linear function of the n transportation system attributes,  Xk  k=1,n
Vit  = a0t + a1t X1 + a2t X2 + a3t X3 + … + ant Xn
Where
akt = are the calibrated (possibly individual traveler-specific (t)) coefficients of the linear utility model.
For two modes:  If U* is the difference in utility between two modes, U*  =  UA  - UB   ,   then

PB = e -U* /  ( 1 + e -U*  )  and   PA = 1  /  ( 1 + e -U*  )
Page 1 of 1 
10/16/07
Orf 467 Fall ‘07
Page 2 of 2 
10/16/07
Alain L. Kornhauser

